STATE OF TEXAS			§

COUNTY OF CHAMBERS 	§

A METES & BOUNDS description of a certain 94.91 acre tract of land situated in the Jacob Townsend Survey, Abstract No. 25 in Chambers County, Texas, being out of the remainder of a called 154.388 acre tract of land conveyed to Angle Brother Properties, LLC by Special Warranty Deed with Vendor's Lien recorded in Clerk's File No. 93341 of the Chambers County Official Public Records (CCOPR) and all of the called 4.40 acres of land being Restricted Reserve “B” of the Angel Brothers Reserve Minor Replat recorded in Document No.  said 94.91 acre tract being more particularly described as follows with all bearings being based on the Texas Coordinate System of 1983, South Central Zone;

BEGINNING at a found 5/8-inch iron rod being the northwest corner of said 154.388 acre tract being common with the southwest corner of tract 6 & 7 conveyed to Cowboy’s Service Inc by Special Warranty Deed with Vendor’s Lien recorded in Clerk’s File No. 81949 CCOPR, and being in the east line of a called 40.53 acre tract of land conveyed to Angel Brothers Enterprises, Inc by Warranty Deed recorded in Volume (01)492, Page 326 CCOPR;

THENCE, North 87°04'16" East, along the north line of the called 154.388 acre tract (common with the north line of the Jacob Townsend Survey, Abstract 25), being common with the south line of said Tract 6 & 7, the south line of a called 59.30 acre tract of land conveyed to West Chambers Development Corporation by General Warranty Deed recorded in Volume (02)567, Page 784 CCOPR, passing at 1264.25 feet (0.14 feet left) a found 1/2-inch iron rod with cap (stamped “CHANDLER”) marking the southeast corner of said 59.30 acre tract, continuing along common line being the south line of called 10.65 acre tract conveyed to Emidio & Carmela Martinez by Special Warranty Deed with Vendor's Lien recorded in Clerk's File No 76951 CCOPR, the south line of a second called 10.65 acre tract of land conveyed to Emilio & Carmela Martinez by Special Warranty Deed with Vendor's Lien recorded in Clerk's File No. 75157 CCOPR to a found 2-inch iron pipe for corner marking the southeast corner of said 10.65 acre tract being common with the southwest corner of a called 20.72 acre tract as described in General Warranty Deed recorded in Volume (04)685, Page 596 CCOPR, for a total distance of 1773.49 feet;

THENCE, North 87°15'45" East, along the north line of the called 154.388 acre tract (common with the north line of the Jacob Townsend Survey, Abstract 25), being common with the south line of said 20.72 acre tract, passing at 497.36 feet (0.24 feet left) a found 3/4-inch iron rod marking the southwest corner of a called 10.363 acre tract as described in Warranty Deed recorded in Volume (00) 455, Page 454 of the CCOPR, from which a found 1/2-inch iron rod bears South 01°13'45” East, 1.73 feet marking the same corner, continuing along the north line of the called 154.388 acre tract passing at 744.71 feet (0.30 feet left) a found 5/8-inch iron rod marking the southeast corner of said 10.363 acre tract and the southwest corner of a called 5.19 acre tract as described in Special Warranty Deed recorded in Volume (07) 914, Page 044 of the CCOPR, continuing along the common line of said 5.19 acre tract and the called 154.388 acre tract in total a distance of 763.87 feet to a found 3/4-inch iron rod with cap stamped “Centerpoint Energy” marking the northwest corner of a called 10.02 acre tract of land conveyed to CenterPoint Energy Houston Electric, LLC by Special Warranty Deed recorded in Clerk’s File No. 2025-214234 CCOPR;

THENCE, over and across said 154.388 acre tract, along the west line of said 10.02 acre tract of land the following five (5) courses and distances:

- South 00°00'00" West, 445.02 feet to a found 3/4-inch iron rod with cap stamped “Centerpoint Energy”;

- South 90°00'00" West, 825.06 feet to a found 3/4-inch iron rod with cap stamped “Centerpoint Energy”; at the beginning of a curve to the right;

- Along the arc of said curve to the right having a radius of 25.00 feet, a central angle of 40°07'10", an arc length of 17.51 feet, and a long chord bearing North 69°56'26" West, 17.15 feet to a found 3/4-inch iron rod with cap stamped “Centerpoint Energy” at the beginning of a reverse curve to the left;

- Along the arc of said reverse curve to the left having a radius of 60.00 feet, a central angle of 40°05'20", an arc length of 41.98 feet, and a long chord bearing North 69°55'26" West, 41.13 feet to a found 3/4-inch iron rod with cap stamped “Centerpoint Energy”;

- South 00°15'36" West, 60.00 feet to a found 5/8-inch iron rod with cap stamped “JONES|CARTER”;

THENCE, North 90°00'00" East, 50.00 feet to a point for corner, from which a found 3/4-inch iron rod with cap stamped “Quiddity Eng. Property Corner” being the northeast corner of Restricted Reserve “A” of said Angel Brothers Reserve Minor Replat bears North 90°00’00” East, 1605.08 feet;

THENCE, South 00°03’34” West, over and across said Restricted Reserve “B”, passing a found 3/4-inch iron rod with cap stamped “Quiddity Eng. Property Corner” at 70.00 feet, then along the west line of said Restricted Reserve “A”, common with the east line of said Restricted Reserve “B”,  1053.10 feet to a set 3/4-inch iron rod with cap stamped “Quiddity Eng. Property Corner at the beginning of a non-tangent curve to the left;

THENCE continuing along said west line of Restricted Reserve “A” being common with the east line of said Restricted Reserve “B” the following four (4) courses and distances:

- Along the arc of said non-tangent curve to the left having a radius of 1833.61 feet, a central angle of 07°18'59", an arc length of 234.15 feet, and a long chord bearing South 06°37'27" East, 233.99 feet to a set 3/4-inch iron rod with cap stamped “Quiddity Eng. Property Corner”

- South 11°34'08" East, 198.87 feet to a set 3/4-inch iron rod with cap stamped “Quiddity Eng. Property Corner” at the beginning of a curve to the left;

- Along the arc of said curve to the left having a radius of 100.00 feet, a central angle of 13°25'53", an arc length of 23.44 feet, and a long chord bearing South 18°17'04" East, 23.39 feet to a set 3/4-inch iron rod with cap stamped “Quiddity Eng. Property Corner”

- South 25°00'00" East, 81.33 feet to a set 3/4-inch iron rod with cap stamped “Quiddity Eng. Property Corner” being the southwest corner of said Restricted Reserve “A” common with the southeast corner of said Restricted Reserve “B” and being in the north line of Kilgore Parkway, 120 feet wide as described in Volume (09)1148, Page 479 CCOPR at the beginning of a non-tangent curve to the right;

THENCE, departing said common line, along the arc of said non-tangent curve to the right being in the north line of said Kilgore parkway, having a radius of 12432.34 feet, a central angle of 08°15'24", an arc length of 1791.56 feet, and a long chord bearing South 71°24'04" West, 1790.01 feet to a found 3/4-inch iron rod with cap stamped “JONES|CARTER” being in the west line of the aforementioned 154.388 acre tract;

THENCE, North 02°39'44" West, along said west line, 2477.79 feet to the POINT OF BEGINNING, CONTAINING 94.91 acres of land in Chambers County, Texas, as shown on Drawing No. 21484 V3 in the office of Quiddity Engineering in Bellaire, Texas.

Quiddity Engineering, LLC

6330 West Loop South, Suite 150			Acting By/Through Steven Jares

Bellaire, Texas 77401					Registered Professional Land Surveyor

(713) 777-5337						No. 5317

Texas Board of Professional Land Surveying			SJares@quiddity.com

Registration No. 10046100